SWV20 Version 1.1

Copyright 1987 Thomas M. Langley

All Rights Reserved

SWV20 ��is �a �driver for the NEC �V-Series �microprocessors �that

enables �the �8 �bit emulation mode of the device and �creates �an

environment �that �emulates a superset of the 8 bit CP/M.  ��SWV20

runs �on any V20 �or V30 �equipped microcomputer �running �C/M-86,

MP/M-86, �Concurrent CP/M-86, Concurrent DOS-86 �or Concurrent PC-

DOS.  �SWV20 �can operate as SW! �in a CompuPro environment and be

automatically invoked �by the system shell during program �search.

The �program �operating �in the 8 bit environment �has �access �to

nearly �all �of �the �function �calls �available �under �the �host

operating system, thus far more than normally available under CP/M

version 2.2.

Installation:

To install SWV20, copy the SWV20.CMD file, the SWV20I.BAT file and

the *.INP �files onto your hard disk or second floppy and log onto

that drive. Next, make sure that sid86 �or ddt86 is present on the

a drive and that �it's �user number is 0 (root directory for a DOS

disk), and enter SWV20I swname [C]

swname is the command name to be used for the new SWV20

use SW! for CompuPro.

C      if entered (the [] means it's optional and should

not be entered) means this is for a CompuPro system.

The batch file will create the file swname.CMD

Now for all systems except CompuPro, �load each 8 bit program onto

your system and change the .COM extension to .CPM.  �For �CompuPro

systems, ��copy the .COM program onto your system and run the 8BIT

utility �supplied with Concurrent DOS-816 �to set the proper �file

attributes.  ��That's all there is to it.  �To run a program enter

the �program name you gave SWV20 �and the CP/M command line.  �For

example: to run the 8 bit program ddt on test.com and you gave the

program the name SWV20 �you would enter -  SWV20 DDT TEST.COM. For

CompuPro systems, �there is no need to enter the SW! �program name

just �enter �DDT �TEST.COM and the system �shell �will �internally

invoke �SW! ��when �it detects that the request is for �an �8 �bit

program.  NOTE for CompuPro Systems: �When setting up the COMPUPRO

switch programs, �assume an 8085/8088 system  UNLESS you are using

SPU-Z �in �which �case �there is no need for �SWV20.  ��There �are

additional �switch �programs �involved if you have �various �slave

processors �installed in your system.  �You can install SWV20 ��in

place �of SW!8085 �that came with your system.  �You can use SWV20

with the CPU-8085/88 board or CPU-8086/87.

#########################################################################################################################