EMUL.DOC	as of 12/07/85		version 2.20

This is version 2.20 of EMUL.CMD, an 8080 emulator for use with the NEC

uPD70116 and uPD70108 series CPUs (also known as the V30 and V20,

respectively).

NOTE:	UPDATE INFO APPENDED TO END OF FILE

This copyrighted program has been released to the Public Domain

for NONCOMMERCIAL uses only. Users wishing to use this utility

for ANY purpose other than personal use must first obtain written

authorization. Commercial distribution of this utility is

strictly forbidden without authorization and a $20.00 royalty fee

per distributed copy.

The above does not apply to distribution via "SIG-M library"!

The author assumes no liability of any kind for damages arising

from the use or inability to use this utility.

Background:

The NEC V20 and V30 are direct hardware replacements for Intel's 8088

and 8086 CPU chips, respectively, and in most cases just drop in and

make life SLIGHTLY faster.  The speed improvement is in many cases nothing

to lose sleep about, certainly not worth the twenty bucks you'll have to

shell out for the chip!  I have run a series of "average" programs and

found that they USUALLY run about 5% faster.  A popular Public Domain

CPU speed test for MSDOS clocked the V30 at 7.85MHz and the 8086 at 7.95!!!!

There are 3 reasons, however, why you might want to spend the twenty anyway.

First, the little critters are 80186/80188 instruction compatible. This means

you can do your PUSHAs and POPAs and your string out a'la Z80's OTIR.  Note

that the V-series, unlike the 80186, does not mask the shift/rotate count,

so there is no easy way of telling the chip is not an 8086/88...

Second, NEC crammed a number of unique instructions into the chip, giving

you basically the same type bit handling like the Z80, as well as BCD

shift/rotate. Here a note to BCD: the manual does not explain, but fact is

that a BCD shift/rotate wipes out the high nybble of AL. As if that mattered,

anyway...

Third, and now we're at the bottom of all THIS, NEC has added a mode bit

which, when reset, makes that chip think it's an 8080 with 4 afterburners.

You'll see shortly why afterburners.

Foreground:

Picture yourself sitting in front of some CP/M-86 or 16 bit TurboDOS machine

wanting, like I have for YEARS now, to be able to run you favorite text

editor which is unavailable outside the CP/M-80 world.  Frustrating that

you have to put up with old WordPlanet or somesuch, where you could, for

example run MINCE (the king of editors, yea!!!) if you only had something to

run it on!!!!

Now, in the TurboDOS world there is a neat Z80 simulator called TZ80 which

runs nearly every program that would ordinarily run under 8 bit TurboDOS.

Unfortunately, it is a SIMULATOR, and instructions have to be decoded one

at a time and executed. It is SLLLLLLLLLOOOOOOOOOOOOOW. Also, aside from

some inferior Public Domain "emulators", there is no such beast available

for CP/M-86 that runs reliably.

Enter EMUL.CMD.

EMUL is a true emulator which only works with NEC's V20 and V30 CPUs and

EXECUTES 8080 instructions. It allows you to run just about ANY CP/M-80

program with the following exceptions:

1. The CP/M-80 program MUST be 8080-only (no Z80 code) or the HCF (halt

and catch fire) instruction will obliterate every byte of RAM!

2. You must be running a NEC CPU, either V20 (uPD70108) or V30 (uPD70116);

EMUL tries to do some checking but may not be able to tell in all cases!

3. Your 8080 program must not attempt any BIOS calls other than for

console I/O. All other calls are IGNORED.

EMUL has been tested with the following software:

MBASIC 5.21

MINCE 2.6

WordStar 3.3

SID 3.0

M80

LINK-80

BDS C 1.50A

Internals:

To TurboDOS programmers, the source code may be readable. Those used to

ASM-86 or RASM may find it a bit weird. Sorry, that's what I use around

here.   The only real confusion will arise in the fact that in EMUL ver

2, the 8080 environment is compacted so as not to take up 64k of mostly

empty space. Since the environment is ORGed in the data segment and the

8086 data segment includes base page.    Therefore, all offsets used in

the 8080 environment must not only be adjusted for theit actual running

location, but also for their relative location at load time.    This is

why you see equates like "BDOS-0X100",  or in the base page setup code,

"DFCB1-0X100" is used.   End effect is, that EMUL works.   The programs

hardest to please in regard to base page setup were M80 and MBASIC, and

they both work flawlessly now.

The afterburner:

I wrote a timing proram with the following core:

MVI	E,50

LXI	B,0

LOOP:	XTHL

XTHL

DCX	B

MOV	A,B

ORA	C

JNZ	LOOP

DCR	E

JNZ	LOOP

The program, running on a 6MHz Z80 under TurboDOS 1.41 executed

in 35.84 seconds, including time to access the clock.

The same program running on an 8MHz V30 under EMUL executed

in 24.88 seconds, making execution speed equivalent to an 8.64MHz Z80!

I have also run the following speed tests under the following conditions:

Z80 TurboDOS, 6MHz CPU, 800k RAM disk on 16 bit slave

8086 TurboDOS, 8MHz V30, 800k RAM disk on slave

Times:				Z80		V30/EMUL

WordStar: ^Q^C on 180k file:	0:59		0:17

WordStar: ^F last line:		1:05		0:21

M80 assemble 64k source:	1:36		0:59

Compile+link RBBS4 (BDS C):	2:59		1:58

Ever seen ANY 8080 THAT FAST????!!!!!?????

have fun 8080ing!!!!!!!!

-------------------------- UPDATE INFORMATION ---------------------------

VERSION 2.10  11/25/85

CP/M-86 returns a  version number which may be objectionable to some

programs. Therefore, the equate "CPMVER" in the source file has been

added, which should be customized for your system if you do not have

CP/M-86. It is currently set to return a CP/M version of 2.2,  which

should work in most cases.  If you run this under either  MP/M-86 or

TurboDOS, you may want to set it to return 3.0 and/or MP/M.

To do this, load EMUL.CMD into a debugger (in edit mode,  not trace)

and change the word at DS:6AF from 22 00 to 30 01.   The low byte is

the version number and the high byte is the MP/M flag (0 or 1).

NOTE that EMUL simply routes  ALL other function requests through to

the host operating system, so no further changes are required to run

programs that do MP/M record locking.

TurboDOS  users should always set the  CP/M version to 30 01 and add

MPMSUP to the slave GEN file.   TurboDOS users may change the CPMVER

equate and reassemble with TASM and link with TLINK (use -1 option).

VERSION 2.11  11/28/85

Changed default message to show TPA size. Noticed the following:

If the 8080 stack pointer points to an ODD address, all stack

operations will be degraded. If possible, check programs and adjust

stack pointer to point to an EVEN address. Example: A small program

using CALL and RET ran in 6.4 seconds with even stack pointer and

8.68 seconds with odd stack pointer.

VERSION 2.20  12/07/85

Minor cosmetic changes.  Also changed error check after opening file

to work correctly under CP/M-86.  EXPERIMENTALLY trapping the emulator

BDOS calls now since I suspect certain BDOS functions destroy BP

(which serves as SP for the 8080).

#requests through to

the host