FSS Screen Saver

================

Version 1.0 for CP/M-86 - by Kirk Lawrence and Freek Heite.

This program will blank the video screen after a certain time of

keyboard inactivity.

The screen is blanked by manipulating some video hardware registers. Once the

screen has been put into "saving mode", normal processing is resumed (e.g.

a long-running calculation or batchfile will continue with its work).

Running FSS.CMD

===============

The first time you run FSS.CMD, the screen saver program will be placed

in FIDD memory ("made resident") and do its job.

Any subsequent time you run FSS.CMD, the program will not be loaded again.

Instead, the parameter you use for these subsequent invocations will be

passed to the copy of FSS that is already in memory. So you can change the

behaviour of the screen saver without loading multiple copies of the same

program.

Note that you cannot remove the screen saver from memory. However, you can

deactivate it by running FSS again, with parameter D (see below).

FSS Syntax

==========

FSS <required-parameter>

<required-parameter>

is a single character, used to control the time-out of the

screen-saver, and to generate some diagnostic output of the program

FSS.CMD command line parameter

==============================

The program requires a single parameter. Choose one from the following list:

0...9

use a single digit to specify the amount of time (in minutes)

after which the screen saver should start. A value of 0 (zero) will

start the screen saver after about 5 seconds.

V

is an abbreviation for "verbose". This parameter will cause FSS.CMD

to display some diagnostics information that might be helpful for

trouble shooting.

When using this parameter, the screen saver will start after one

minute of keyboard inactivity.

D

puts the screen saver program in Deactivated mode, i.e. the screen

saver will not start. To re-activate, run FSS again with one of the

other parameters.

What you need

=============

1. An IBM or IBM-compatible PC.

2. One of the following video systems:

- VGA

- Hercules monochrome TTL

3. CP/M-86 version 1.1 for the IBM PC and PC/XT, with at least 1 KB of FIDD

memory configured (see below)

FIDD memory

===========

In their version 1.1 of CP/M-86 for the IBM PC and XT, Digital Research added a

way to reserve from 0 to 99 KB of the RAM memory in the PC for use by so-called

"Field Installable Device Drivers" or FIDD programs.

The FSS screen saver program needs 1 KB of this FIDD memory.

To reserve this (or a larger) amount of FIDD storage you should use the

program SETUP.CMD that is part of the CP/M-86 version 1.1 distribution:

- Start the SETUP program

- press function key F5 "Field Inst. Device Driver" to change the amount of

FIDD memory, and follow the instructions on the screen.

Upon exit from the SETUP program, press the appropriate function key to

write the settings to your harddisk (when CP/M-86 was booted from that

harddisk) or to a diskette in drive A:. In the latter case, be sure to put

your CP/M-86 boot diskette in drive A: before committing the changes to disk.

Finally, reboot your CP/M-86 system to activate the new FIDD settings. During

a boot, CP/M-86 will tell you how much FIDD memory is available.

Last but not least

==================

You may use, misuse and abuse this software any way you want - but at your own

risk. The authors of the software do not accept any responsibility for the

consequences of such use, misuse or abuse.

Version: 1.0

Date   : 28Dec2000

Authors: Kirk Lawrence and Freek Heite

email  : fheite@knoware.nl